QS7305 - Evaluation Board Schematic Design

EVB Name:__Si7013/20/21 postage stamps_________

Revision:____1.0__			Date:_2/21/2013___

Revision Control Instructions:

Post Hardcopy Revisions: No

Items for Hardcopy Release: None

# Purpose

This document defines Silicon Laboratories’ evaluation board design procedures.

# Scope

This procedure applies to all evaluation board schematic designs at Silicon Laboratories.  Redesigns are accomplished through the appropriate portions of the design flow.

# Applicable Documents

See documents reference in section 5.

# Responsibilities

The project managers (lead applications engineers) are responsible for making sure that the checklists are checked and completed at the appropriate time in the design cycle.

# Procedure Overview

## Evaluation Board Design Flowchart

## Evaluation Board Schematic Design Flowchart (QS7305)

# Feature Definition

Design Checklist

| Y N | Design to support features / functions listed in chip datasheet |
| --- | --- |
| Y N | Design of support circuits for compatibility with complete evaluation platform: - power supply regulation / conditioning - external PC programming support - input/output/ generation of needed reference signals - enables/triggers to/from external test equipment - compatible connectors |
| Y N (NA) | Design to meet statutory and regulatory requirements |
| Y N | Include marketing / customer feature requirements |
| Y N | Pin-by-pin review of each IC to ensure proper electrical interface |
| Y N N/A | Provide EVB test software plan and requirements to Software Manager |
| Y N N/A | Provide EVB applications software plan and requirements to Software Manager |
| Y N | Provide EVB build plan to Engineering Services |

Project Planning Checklist

| Y N | Determine schedule requirements |
| --- | --- |
| Y N N/A | Provide list of long lead time components to be ordered |
| Y N | Obtain approved landing patter for Silabs devices from Packaging team |

Note features requiring additional definition/ responsibility

Approvals/Date:

Project Manager: _____John Gammel___________   Date :___2/20/2013_____

(Project manager shall retain a copy for the life of the product)

Applications Manager:          Colin Tompkins                  Date :_2/20/2013____

Marketing Manager:                  Jim Judkins             Date :_2/20/2013___

Design:______Bruce Del Signore_____ Date :___2/20/2013______

Software Manager:____________NA_______ Date :_________

Engineering Services:            Renata Hall         Date :_________

Applications Director:             Paul Brown         Date :_________

## New Design Generation

| Y N | Generate new directories for design (see section 9.0) |
| --- | --- |
| Y N | Schematic capture done using OrCAD CIS Windows |
| Y N | Use schematic page template from \\silabs.com\apps\orcad\Si_TEMPLATE.DSN |
| Y N | Enter evaluation board name in title block |
| Y N | Set revision number to 1.0 |
| Y N | Add revision block / page showing initial creation |
| Y N N/A | Include parts library from the CIS database |
| Y N N/A | Add new parts to CIS database |
| Y N Y N Y N Y N N/A Y N N/A Y N N/A Y N N/A Y N N/A Y N N/A | Verify parts used have the following fields: - Value - PCB Footprint - Tolerance - Rating - Material - Manufacturer - Manufacturer part number - other fields as needed |
| Y N N/A | Add PCB silkscreen labels in quotes and “Silk” property where applicable |
| Y N N/A | Include layout-critical information (printed trace specifications, PCB stack up) |
| Y N N/A | Organize layout for easy export to datasheet application schematic |
| Y N N/A | Save file in official location (see section 9.0) |

## Design Revision Generation

| Y N | Generate new directories for revision (see section 9.0) |
| --- | --- |
| Y N N/A | Use previous revision schematic as template |
| Y N N/A | Increment revision number to x.y |
| Y N N/A | Update revision block / page with changes made |
| Y N N/A | Create any new parts in CIS database |
| Y N Y N Y N Y N N/A Y N N/A Y N N/A Y N N/A Y N N/A Y N N/A | Verify parts used have the following fields: - Value - PCB Footprint - Tolerance - Rating - Material - Manufacturer - Manufacturer part number - other fields as needed |
| Y N N/A | Add PCB silkscreen labels in quotes and add “Silk” property where applicable |
| Y N N/A | Update PCB board number as part in schematic if layout changes to be made |
| Y N N/A | Update layout-critical information (printed trace specifications, PCB stackup) |
| Y N N/A | Organize layout for easy export to datasheet application schematic |
| Y N N/A | Save file in official location (see section 9.0) |

## Design Rules Check (DRC)

| Y N | Perform design rules check OrCAD menu item Tools - Design Rules Check |
| --- | --- |
| Y N | The following Design Rules Check fields should be selected: - Scope - check entire design - Action - check design rules - Report - Create DRC markers for warnings - Check off-page connector connections - Report identical part references - Report invalid packaging - Check unconnected nets |
| Y N | Use ERC Matrix defaults |

## Netlist Generation

| Y N | Use format padspcb.dll from OrCAD menu item Tools - Create Netlist - Other - padspcb.dll |
| --- | --- |
| Y N | Verify netlist is created with no errors |

## Bill of Materials (BOM) Generation

| Y N | Perform bill of materials generation using Orcad menu item Tools - Bill of Materials |
| --- | --- |
| Y N N/A | Use bill of materials header and combined property string in \\silabs.com\apps\CIS\Setup\BOM string.txt |
| Y N | The following Bill Of Materials fields should be selected: - Scope - process entire design - Line item definition - Header and Combined Property String should be copied from \\silabs.com\apps\CIS\Setup\BOM string.txt - Use default settings for all other fields |
| Y N | Create bill of materials in Microsoft Excel format - Run BOM macro located in \\silabs.com\apps\CIS\Setup\BOMMacro.xls - Adjust fields as needed |

# Schematic Review / Finalization Checklist

## Schematic Review

| Y N | Conduct a formal schematic review evaluating functionality and design - include key product team members and at least 2 members from another product team (done by email) |
| --- | --- |
| Y N | Verify all connections of the schematic are as intended |
| Y N | Verify net list Power and Ground connections are as intended |
| Y N | Review Excel Bill of Materials for part description accuracy |

## Finalization

| Y N Y N Y N | Generate .pdf summary file to include: - schematic pages - bill of materials from Excel spreadsheet |
| --- | --- |
| Y N N/A | Ensure all files saved in official directory (see section 9.0) |

# EVB Datasheet

| Y N N/A | Initialize / revise evaluation board datasheet in conjunction with Marcom group |
| --- | --- |
| Y N N/A | Evaluation board datasheet should include: - schematic - description of options and jumpers, including default settings - bill of materials - connector / header description |
| Y N N/A | Save draft in official directory, to be revised after layout and software complete |

# Filename conventions and directories

## Suggested filename convention

Files should be saved with the naming convention:

<board name>_rx_y.xxx

where:

<board name> 	= EVB name

rx_y		= numerical revision x.y

xxx		= extension for file format (.pdf, .dsn, .bom, etc)

examples:

Si3241DC5-EVB_r1_3.dsn

Si3241DC5-EVB_r1_3.pdf

## Suggested Directory Structure

\\silabs.com\apps\wireline\<product line>\EVB\<board name>

\rx_y

\schematic

\layout

\doc

\app_software\<program name>\rx_y

\src

\dist

\test_procedure

\test_reports

Approvals/Date:

Project Manager: ______John Gammel_______   Date :__2/20/2013____

(Project manager shall retain a copy for the life of the product)

Engineering Services:            Renata Hall                Date :__________

(Approval required prior to beginning layout)

Peer Member: _____Morrie Altmej____________   Date :_2/20/2013___

(Peer member present during schematic review)

Peer Member: __Colin Tompkins______   Date :___2/20/2013______

(Peer member present during schematic review)

Design______Bruce Del Signore_________ Date :___2/20/2013__

Document Revision History

| Rev | Date | #### Initiator | Description of Changes |
| --- | --- | --- | --- |
| A | 1/28/2000 | Craig T. Phillips | Initial release of document. |
| B | 4/25/00 | H. Robertson | Reformat of Header. |
| C | 12/11/02 | Chris McCrank | Revision to include schematic entry only |
| D | 9/25/06 | Bud Welch | Obsolete document |
| E | 7/16/2007 | Renata Kudlek | Reactivate Obsolete version and Revise for release |
| F | 7/16/2008 | Renata Hall (Kudlek) | Directory structure updated, added place for board name and revision to front page. Also, change doc number from QS0422 to QS7305 to match ISO9000 TS16949 paragraph numbering & correct refs to the old numbers in the text of the doc. |
| G | 8/21/2009 | Renata Hall | Updated to reflect CIS based changes to schematic generation. Changes include new requirements for creating and selecting parts from the CIS database as well as updated instructions for BOM generation |
| H | 7/28/2010 | Renata Hall | Document updated to require Design approval for EVBs. Requirements for approved landing patter from packaging team also added to section 6 design checklist |
| I | 7/28/2011 | Chuck Tidwell Renata Hall | Changed schematic template filename (6.1), removing spaces and version info. Fixed a typo referencing section (should have been section 9.0) |